# Peak Energy — Corporate Network Architecture

Firewall: PEAK-CORP-FW (OPNsense)  ·  Generated 2026-10-22

This document describes the live network: VLANs, WAN links, Omada Wi‑Fi, biometrics and Keka, WireGuard (Peak Energy VPN), and access policy for PeakPulse.

# Architecture diagram

# 1. WAN links

| WAN | Public IP | Gateway name | Notes |
| --- | --- | --- | --- |
| Airtel Fiber | 125.19.224.18/30 | WAN_AIRTEL | Primary; WG endpoint default |
| Jio RF | 47.247.169.94/30 | WAN_JIO | Failover / alternate WG endpoint |
| WAN_LB | — | WAN_LB | Dual-WAN load balance + failover |

# 2. VLANs / subnets

| VLAN / Role | Subnet | Gateway (FW) | DHCP pool | Interface |
| --- | --- | --- | --- | --- |
| MGMT (admin gear) | 10.80.99.0/24 | 10.80.99.1 | 10.80.99.101–200 | lan / re0 |
| WIRED_CORP | 10.80.100.0/24 | 10.80.100.1 | 10.80.100.101–200 | opt2 / VLAN 100 |
| WL_CORP (PETCPL) | 10.80.101.0/24 | 10.80.101.1 | 10.80.101.101–200 | opt3 / VLAN 101 |
| WL_GUEST | 10.80.102.0/24 | 10.80.102.1 | 10.80.102.101–200 | opt4 / VLAN 102 |
| WireGuard VPN | 10.80.200.0/28 | 10.80.200.1 | static peers | opt6 / wg0 |

# 3. MGMT hosts (10.80.99.0/24)

| IP | Hostname | Role |
| --- | --- | --- |
| 10.80.99.1 | PEAK-CORP-FW / OpnSense | Firewall GUI :4444, SSH, Unbound DNS |
| 10.80.99.2 | sg2428p | Omada switch SG2428P |
| 10.80.99.3 | oc200 | Omada OC200 controller https://10.80.99.3/ |
| 10.80.99.21 | eap670-1 | Wi-Fi AP EAP670 |
| 10.80.99.22 | eap670-2 | Wi-Fi AP EAP670 |
| 10.80.99.23 | eap670-3 | Wi-Fi AP EAP670 |
| 10.80.99.24 | eap225-3 | Wi-Fi AP EAP225 |

# 4. WIRED_CORP hosts (10.80.100.0/24)

| IP | Hostname | Role |
| --- | --- | --- |
| 10.80.100.1 | — | WIRED_CORP SVI / DNS |
| 10.80.100.10 | biometric-server | ACS / Attendance SQL |
| 10.80.100.11 | biometric-entry | Biometric entry terminal |
| 10.80.100.12 | biometric-exit | Biometric exit terminal |
| 10.80.100.21 | laser-printer | HP LaserJet (Ethernet) |
| 10.80.100.51 | peak-ws-1 | Workstation 1 |
| 10.80.100.52 | peak-ws-2 | Workstation 2 |
| 10.80.100.53 | peak-ws-3 | Workstation 3 |
| 10.80.100.54 | dev-ws-1 | PeakPulse / peakpulse-dev.peakenergy.asia |
| 10.80.100.55 | peak801-in-conf | Conference room PC |

# 5. WL_CORP hosts (10.80.101.0/24)

| IP | Hostname | Role |
| --- | --- | --- |
| 10.80.101.1 | — | WL_CORP SVI / DNS |
| 10.80.101.21 | laser-printer | HP LaserJet (Wi-Fi) |
| 10.80.101.22 | printer-md | MD HP Smart Tank Wi-Fi |
| 10.80.101.30 | tv-conferance | TV Conference Room |
| 10.80.101.31 | tv-kalaam | TV Abdul Kalaam |
| 10.80.101.32 | tv-ratan | TV Ratan Tata |
| 10.80.101.33 | tv-onm | TV Operations & Mgmt |
| 10.80.101.34 | tv-md | TV MD |

# 6. WireGuard / Peak Energy VPN

The Windows client is Peak Energy VPN. On PETCPL or wired corp the tunnel must be off, because AllowedIPs sends 10.80.0.0/16 into WireGuard.

| Peer | Tunnel IP | Access |
| --- | --- | --- |
| PEAK-WG server | 10.80.200.1/28 | OPNsense wg0 listen UDP 51820 |
| binny.andrews | 10.80.200.2/32 | Admin: FW GUI/SSH + MGMT + CORP |
| admin | 10.80.200.3/32 | Admin: FW GUI/SSH + MGMT + CORP |
| jagadeshwar | 10.80.200.4/32 | AMC: CORP .100/.101; RDP allowed |
| venu.gopal.reddy | 10.80.200.5/32 | AMC: CORP only; RDP blocked |
| poovarasu | 10.80.200.6/32 | AMC: CORP only; RDP blocked |
| sanoj.james | 10.80.200.7/32 | AMC: CORP only; RDP blocked |
| sampada | 10.80.200.8/32 | AMC: CORP only; RDP blocked |
| sharan | 10.80.200.9/32 | AMC: CORP only; RDP blocked |

# 7. Biometrics and Keka

Attendance stays on WIRED_CORP. Entry and exit terminals talk to the ACS on the same VLAN, so that traffic does not pass the firewall. Keka cloud pulls SQL from the public WAN address on TCP 1433. NAT on Airtel and Jio forwards that only from KEKA_HR to 10.80.100.10, the active ACS. The old Akrivia source alias is kept and disabled.

| Host | Address | Role today |
| --- | --- | --- |
| biometric-server | 10.80.100.10 | Active ACS. Alias ACS_SERVER. Keka SQL target. |
| biometric-entry | 10.80.100.11 | Entry terminal. Same VLAN as ACS. |
| biometric-exit | 10.80.100.12 | Exit terminal. Same VLAN as ACS. |

# 8. Internal DNS (Unbound)

| Name | Address | Use |
| --- | --- | --- |
| fw.peakenergy.asia | 10.80.99.1 | Firewall (MGMT) |
| peak-corp-fw.peakenergy.asia | 10.80.99.1 | Firewall (MGMT) |
| fw-corp.peakenergy.asia | 10.80.100.1 | Firewall (WIRED_CORP) |
| acs.peakenergy.asia | 10.80.100.10 | Attendance / ACS SQL |
| oc200.peakenergy.asia | 10.80.99.3 | Omada controller |
| app.peakenergy.asia | 10.80.100.54 | PeakPulse |
| peakpulse-dev.peakenergy.asia | 10.80.100.54 | PeakPulse |

# 9. Security and access policy

| Topic | Current policy |
| --- | --- |
| PeakPulse public WAN | DISABLED — HTTP/HTTPS DNAT kept but turned off on both WANs |
| PeakPulse access | On-site LAN or WireGuard only → 10.80.100.54 (dev-ws-1) |
| WG DNS | New peers use 10.80.200.1 (Unbound on WIREGUARD). Legacy may use 10.80.100.1. Both are allowed. |
| WG AllowedIPs | 10.80.0.0/16 split tunnel (corp only, not full internet) |
| WG endpoints | 125.19.224.18:51820 (Airtel, default) or 47.247.169.94:51820 (Jio) |
| WG interface | opt6 / wg0, description WIREGUARD. Routes disabled; firewall does the policy. |
| MGMT via VPN | Only WG_FW_ADMINS (binny.andrews .2 and admin .3) |
| FW GUI / SSH via VPN | Only WG_FW_ADMINS. GUI listens on MGMT, WIRED, WL_CORP, and WIREGUARD — never WAN. |
| AMC via VPN | CORP only: 10.80.100.0/24 and 10.80.101.0/24. No MGMT, no firewall GUI. |
| RDP over VPN | TCP 3389 blocked for Venu, Poovarasu, Sanoj, Sampada, Sharan. Allowed for binny, admin, Jagadeshwar. |
| ACS SQL from Internet | KEKA_HR only (cin3.a.keka.com) → 10.80.100.10:1433 on both WANs |
| Akrivia | AKRIVIA_HR alias kept, disabled |
| Corp Wi-Fi MAC list | WL_CORP_MACS rules exist but are DISABLED. PETCPL is open to the VLAN. |
| Guest Wi-Fi | SSID PETCPL-GUEST, VLAN 102, captive portal. Internet Mon–Fri 07:00–19:00. No internal access. |
| On-site vs remote | On PETCPL or wired corp, Peak Energy VPN OFF. Remote, VPN ON. |

# 10. Logical traffic paths

On-site (PETCPL / wired): clients use the VLAN gateway. Peak Energy VPN stays OFF so 10.80.x is not forced into the tunnel.

Remote: Peak Energy VPN ON, then UDP 51820 to PEAK-WG. AMC peers reach CORP (.100 and .101), including PeakPulse and the ACS. Admins also reach MGMT (.99) and the firewall GUI. RDP is blocked for every AMC peer except Jagadeshwar.

Keka: cin3.a.keka.com to WAN TCP 1433, then 10.80.100.10.

Guest: PETCPL-GUEST on VLAN 102, captive portal, blocked from MGMT and CORP, internet only on the weekday schedule.

Confidential — Peak Energy IT. Live PEAK-CORP-FW policy and WireGuard peer pack. Generated 2026-10-22.