PEAK Corporate Network

Landscape, Devices, Attendance Integration &
FortiGate → OPNsense Migration Guide

Source config: PEAK-CORP-FW_7-2_1762_202609160923.conf.yaml
Appliance: FortiGate 100F · FortiOS 7.2.13 build 1762
Document generated for migration planning

# Contents

1. Executive summary

2. Current network landscape (visual)

3. Device inventory

4. VLAN and addressing plan

5. Dual WAN and SD-WAN

6. Firewall policies and NAT

7. Biometric attendance, ACS, Akrivia and Keka

8. Wireless and FortiSwitch

9. Remote access (SSL VPN → WireGuard)

10. Security findings

11. OPNsense target architecture

12. Cutover checklist

13. File annex (migration kit)

# 1. Executive summary

PEAK operates a FortiGate 100F (hostname PEAK-CORP-FW) as the edge firewall with dual ISP connectivity (Airtel fiber and Jio RF), a FortiSwitch core (PEAK-CORE-SW), and three FortiAP 431F access points. Corporate wired and wireless networks use VLANs 100–102. An attendance/ACS server at 10.80.100.10 on the wired corporate VLAN stores biometric punch data; that database is exposed to the Internet on TCP 1433 so an external HR stack (Akrivia named in the firewall; Keka used operationally) can pull attendance.

OPNsense can replace the FortiGate as router/firewall, but cannot run FortiLink or FortiAP controller functions. The FortiSwitch must be converted to standalone and wireless APs must be replaced or re-homed before a full cutover. A migration kit under opnsense-migration/ provides aliases, DHCP maps, NAT/firewall mapping, WireGuard peers, and a fill-in-the-NICs seed XML.

# 2. Current network landscape

Figure 1 shows ISPs, FortiGate, switch, major device classes, and cloud HR exposure.

# 3. Device inventory

## 3.1 Core infrastructure

| Role | Device | Identity / addressing | Notes |
| --- | --- | --- | --- |
| Firewall | FortiGate 100F | PEAK-CORP-FW | Admin HTTPS :4444 |
| Core switch | FortiSwitch 148F | PEAK-CORE-SW / S148FFTF23020423 | FortiLink managed |
| AP | FAP-431F | AP-001 FP431FTF23099DUY | Remote WTP |
| AP | FAP-431F | AP-002 FP431FTF23099DJ8 | Remote WTP |
| AP | FAP-431F | AP-003 FP431FTF23099DHU | Remote WTP |
| Staging | OPNsense | MAC 0e:51:d8:1c:e2:01 | Already present as FG object |

## 3.2 Servers and key hosts (VLAN 100)

| Host | IP | Role |
| --- | --- | --- |
| ACS / attendance | 10.80.100.10 | Biometric DB; SQL 1433 + RDP 3389 published |
| OFFICE_PC / PEAK-APP | 10.80.100.103 | HTTP/HTTPS VIP target |
| Clone of OFFICE_PC | 10.80.100.104 | Secondary host object |
| PEAK-WS-1 | 10.80.100.101 | DHCP reservation 4c:d7:17:a4:2a:d0 |
| PEAK-WS-2 | 10.80.100.102 | DHCP reservation 4c:d7:17:a4:2b:11 |

## 3.3 Endpoints called out in config

Two biometric attendance machines are connected to the FortiSwitch (access VLAN 100). They are not individually named as firewall address objects; they reach the ACS server on the same LAN. Staff devices appear as MAC address objects inside the MAC WHITELIST group used by the wired captive-portal exempt list. Printers, meeting-room TVs, conference PC, and TrueNAS are also represented as MAC objects.

# 4. VLAN and addressing plan

Inter-VLAN allow rules exist only between WIRED-CORP and WL-CORP. Guest, ACS-VLAN, CCTV, and default VLAN have no east-west permits beyond what same-interface routing would require—and ACS/CCTV VLANs are unused on the switch today.

# 5. Dual WAN and SD-WAN

| Member | Interface | Address | Gateway | Role |
| --- | --- | --- | --- | --- |
| 2 (preferred) | wan1 Airtel-FIBER | 125.19.224.18/30 | 125.19.224.17 | Priority in LB service |
| 1 | wan2 JIO-RF | 47.247.169.94/30 | 47.247.169.93 | Second LB member |

SD-WAN service Default_Internet_LB uses load-balance mode with SLA Default_DNS (latency 250 ms, jitter 50 ms, loss 5%). Default route uses sdwan-zone virtual-wan-link. On OPNsense this becomes gateway group WAN_LB with monitoring to 8.8.8.8 / 1.1.1.1.

# 6. Firewall policies and NAT

## 6.1 IPv4 policies

| ID | Name | From → To | Action / NAT | Notes |
| --- | --- | --- | --- | --- |
| 8 | LAN > WAN | VLAN 1/100/101/102 → SD-WAN | Accept + NAT | UTM + PEAK-POLICY webfilter |
| 6 | WIRED → WL-CORP | 100 → 101 | Accept | Inter-VLAN |
| 7 | WL-CORP → WIRED | 101 → 100 | Accept | Reverse of 6 |
| 4 | SSL-VPN ADMIN | ssl.root → LAN VLANs | Accept | Group PEAK-VPN-ADMIN |
| 5 | SSL-VPN USER | ssl.root → 100/101 | Accept | Group PEAK-VPN-STAFF (empty) |
| 9 | Akrivia_HR>LAN | WAN → ACS VIPs | Accept + NAT | 1433/3389; src=all |
| 10 | WAN > WiredCorp | WAN → APP VIPs | Accept | 80/443 to .103 |

## 6.2 VIP / port forwards

| VIP | External | Port | Internal |
| --- | --- | --- | --- |
| PEAK-APP-1-Airtel / Jio | Both WAN IPs | 80 | 10.80.100.103 |
| PEAK-APP-2-Airtel / Jio | Both WAN IPs | 443 | 10.80.100.103 |
| ACS-SERVER-Airtel / Jio | Both WAN IPs | 1433 | 10.80.100.10 |
| RDP of ACS-SERVER-* | Both WAN IPs | 3389 | 10.80.100.10 |

# 7. Biometric attendance, ACS, Akrivia and Keka

## 7.1 How it works on site

The two biometric machines connect to the FortiSwitch on normal access ports (VLAN 100). Punch data is sent to the ACS/attendance application on 10.80.100.10. Because devices and server share the same VLAN, this traffic does not require a FortiGate firewall policy.

## 7.2 How external HR pulls data

The FortiGate publishes Microsoft SQL Server port 1433 from both public WAN addresses to 10.80.100.10. Policy Akrivia_HR>LAN permits that path (and RDP 3389). An address object Akrivia_HR (34.87.189.127) exists but is not used as the policy source—the source is all.

## 7.3 Where Keka fits

Keka does not appear anywhere in the FortiGate configuration. Operationally, Keka is the HR application that consumes attendance. That usually means either (a) Akrivia/ACS middleware syncs to Keka, or (b) a Keka connector reaches the on-prem SQL database through the published VIP. Confirm the exact connector URL/IP inside Keka admin and the ACS vendor console—those settings are outside this firewall dump.

## 7.4 Unused ACS VLAN

Interface ACS-VLAN (VLAN 103, 10.80.103.0/24) is defined on the FortiGate, but switch ports are not assigned to it. Do not assume biometrics use VLAN 103 unless you change switch config later.

# 8. Wireless and FortiSwitch

| SSID | VLAN | Schedule | Notes |
| --- | --- | --- | --- |
| PETCPL | 101 | always | Corporate Wi‑Fi, local-bridging |
| PETCPL-GUEST | 102 | peak_optimised Mon–Fri 07:00–19:00 | Guest; passphrase encrypted in dump |

Switch port summary: 1–40 untagged VLAN 100; 41–42 trunks (allowed-vlans-all) for APs; 43–52 untagged VLAN 1. Converting the switch to standalone is mandatory before OPNsense cutover.

# 9. Remote access

FortiGate SSL VPN listens on wan1 and wan2 with pool 10.80.200.1–10. PEAK-VPN-ADMIN members are binny.andrews and admin (full LAN including ACS/CCTV). PEAK-VPN-STAFF has no members. The migration kit replaces this with WireGuard on 10.80.200.0/28 (client configs for both admins). OpenVPN remains an option if FortiClient must be retained.

# 10. Security findings

| Finding | Risk | Recommendation |
| --- | --- | --- |
| SQL 1433 open to Internet on both WANs | High | Restrict to Keka/Akrivia source IPs or VPN only |
| RDP 3389 open to Internet on both WANs | Critical | Remove public RDP; use WireGuard |
| WAN allowaccess https ssh | High | Do not replicate on OPNsense; LAN/MGMT only |
| Akrivia policy src=all despite named object | High | Use AKRIVIA_HR / vendor IP alias |
| Guest not isolated from corp by policy | Medium | Add deny guest → RFC1918 on OPNsense |
| FortiGuard / cert-inspect not portable | Info | Unbound DNSBL + optional Zenarmor |

# 11. OPNsense target architecture

Suggested OPNsense roles: wan=Airtel, opt1=Jio (WAN2), LAN parent with VLAN interfaces opt2–opt7, opt8=WireGuard. Import aliases.csv, apply NAT/firewall from nat-and-firewall.md, edit nics.env and run build-opnsense-seed.py for peak-seed.xml.

# 12. Cutover checklist

Stage OPNsense on spare NIC/VLAN; import aliases; configure WAN_LB and VLANs.

Convert PEAK-CORE-SW to standalone; set trunk to OPNsense; static mgmt on VLAN 100.

Replace or re-home FortiAPs (PETCPL→101, GUEST→102).

Recreate DHCP pools and reservations; verify ACS 10.80.100.10 and APP .103.

Recreate port forwards; lock 1433 to vendor IPs; do not publish 3389/GUI.

Bring up WireGuard; test binny.andrews and admin tunnels.

Confirm biometric punches still land on ACS; confirm Keka/Akrivia sync after VIP move.

Move switch uplink from FortiGate port12 to OPNsense LAN; keep FortiGate powered until verified.

Decommission FortiGate WAN allowaccess and SSL VPN when stable.

# 13. File annex — migration kit

| Path | Purpose |
| --- | --- |
| opnsense-migration/README.md | Migration overview |
| opnsense-migration/nics.env | Fill real NIC names |
| opnsense-migration/build-opnsense-seed.py | Builds peak-seed.xml |
| opnsense-migration/aliases.csv | Firewall aliases import |
| opnsense-migration/mac-whitelist.csv | Captive portal / MAC list |
| opnsense-migration/dhcp-static-maps.csv | DHCP reservations |
| opnsense-migration/nat-and-firewall.md | NAT + rule order |
| opnsense-migration/switch-and-wifi.md | Switch/AP conversion |
| opnsense-migration/wireguard/ | Server keys + client confs |
| opnsense-migration/docs/images/ | Diagram PNGs used in this document |

End of document. Diagrams are also stored as PNG files alongside this Word file for reuse in presentations.